1. Introduction

1.1. Purpose

This document describes the design and architecture of the I2C driver module, based on the requirements defined in the SRS-I2C_Driver. It provides details on data structures, register-level operations, control flow, and the public API used by higher-level modules (in particular the TCS3472 sensor driver). The intent is to give any developer reading this document enough context to understand, maintain, extend, or replicate the driver without having to reverse-engineer the source code.

The driver targets the STM32F411RE microcontroller mounted on the NUCLEO-64 development board, and exposes a hardware-abstracted API that hides the manipulation of the I2C peripheral registers (CR1, CR2, OAR1, OAR2, DR, SR1, SR2, CCR, TRISE, FLTR) and the relevant clock-enable bits of RCC_APB1ENR from the rest of the application.

1.2. Scope

The I2C driver is a low-level software component for embedded systems running on the STM32F411RE. It abstracts the I2C hardware registers documented in RM0383 §18 and provides a clean, uniform API for higher-level modules (such as the TCS3472 color-sensor driver in this lab) to initialize the peripheral, configure the bus speed, and read or write data to a device, either to a specific internal register or as a raw byte stream, without ever touching a register directly.

The driver supports the three I2C peripherals present on the F411 (I2C1, I2C2, I2C3) through an instance pointer, standard mode (100 kHz) and fast mode (400 kHz) bus speeds, and 7-bit addressing. It is built without relying on STM32 HAL or any pre-made abstraction layer; the only header dependency outside of standard C is the device-specific register layout declared inside i2c_driver.h.

1.3. Definitions, Acronyms, and Abbreviations

| Term | Definition |
| --- | --- |
| I2C | Inter-Integrated Circuit. A two-wire (SCL clock, SDA data) serial bus where each device has an address and the master initiates every transaction. |
| MCU | Microcontroller Unit. The STM32F411RE in this project. |
| RCC | Reset and Clock Control. The peripheral block that gates the clock to every other peripheral on the chip. |
| APB1 | Advanced Peripheral Bus 1. The internal bus the I2C peripherals are connected to. |
| SCL / SDA | The two I2C bus lines: SCL is the clock driven by the master, SDA is the bidirectional data line. |
| START / STOP | The bus conditions that begin and end an I2C transaction. A repeated START (Sr) begins a new transaction without releasing the bus. |
| ACK / NACK | Acknowledge / Not Acknowledge. After each byte, the receiver pulls SDA low (ACK) or leaves it high (NACK) to signal whether it accepted the byte. |
| CR1 / CR2 | Control Registers. CR1 holds PE, START, STOP, ACK, SWRST; CR2 holds the peripheral input clock frequency in MHz. |
| CCR | Clock Control Register. Sets the SCL clock period for the chosen bus speed. |
| TRISE | Maximum rise-time register. Configures the SCL rise time for the bus speed. |
| SR1 / SR2 | Status Registers. Hold the event and error flags (SB, ADDR, BTF, TXE, RXNE, BERR, AF, ARLO, BUSY, etc.). |
| DR | Data Register. Writing queues a byte for transmission; reading returns a received byte. |
| SB | Start Bit flag, set after a START condition is generated. |
| ADDR | Address Sent flag, set after the slave address is transmitted and acknowledged. |
| BTF | Byte Transfer Finished flag. |
| AF | Acknowledge Failure flag, set when the device sends a NACK. |
| SRS | Software Requirements Specification. |
| SDD | Software Design Document, this document. |
| FR / NFR | Functional / Non-Functional Requirement, as listed in the SRS. |
| API | Application Programming Interface. The set of public functions exposed by the driver. |

1.4. Requirements

All requirements specific to this module are allocated in SRS-I2C_Driver.md, available in the lab repository at https://github.com/MasterCarlosV/MSE_LabCourse/blob/main/SRS/I2C/SRS-I2C_Driver.md. The driver implements functional requirements FR-1 through FR-10 and complies with the non-functional requirements on error handling (NFR-2) and real-time timeouts (NFR-3), with every public function returning a status code of type i2c_status_t.

2. System Architecture

2.1. High-Level Design

The system follows a classic layered architecture, where each layer talks only to the one immediately below it. This separation is what makes the driver reusable: any I2C device, not just the color sensor in this lab, can be driven by writing a thin device driver on top of this one.

- Application Layer, main.c. Reads the sensor through the TCS3472 driver and prints the result over serial. Never accesses I2C registers directly.

- Device Driver Layer, tcs3472_driver.c/.h. Translates sensor operations (read RGBC, set gain) into sequences of I2C register reads and writes. Passes a device address and register down to the I2C driver but never manipulates an I2C register itself.

- I2C Driver Layer, i2c_driver.c/.h. Implements the public API and is the only layer allowed to touch the I2C peripheral registers and the relevant bits of RCC_APB1ENR.

- Hardware Layer, STM32F411RE I2C peripheral registers, mapped at fixed addresses on the APB1 bus (I2C1 base 0x40005400).

Figure 1 shows the dependency graph of the driver header. The I2C driver is included by the TCS3472 sensor driver, which is in turn included by main.c, so the application reaches the I2C subsystem only indirectly, through the sensor driver.

Figure 1. Files that depend on i2c_driver.h. The driver is consumed by the TCS3472 sensor driver and indirectly by main.c.

2.2. Module Interfaces

The public API is defined in i2c_driver.h. It exposes five functions: one for initialization and two read/write pairs (one register-addressed, one raw). Module-level dependencies are minimal, only the C standard headers stdint.h and stddef.h, plus the device register definitions declared in this same header file. There is no dependency on STM32 HAL, CMSIS device headers, or any third-party library.

Figure 2. Headers included by i2c_driver.h.

Figure 3 shows the inclusion graph of main.c, which reaches the I2C driver indirectly through the TCS3472 sensor header. The graph also shows the other modules reused from earlier labs: the serial and UART code, the GPIO driver, and the formatting utilities.

Figure 3. Inclusion graph for main.c. I2C access flows through the TCS3472 sensor header.

3. Detailed Design

3.1. Class Diagram

Although C is not an object-oriented language, the I2C_RegDef_t struct plays the role of a class in this design: it groups together all the registers that belong to a single I2C peripheral and lets us treat the whole peripheral as a single addressable object. Casting any peripheral base address (0x40005400 for I2C1, 0x40005800 for I2C2, etc.) to an I2C_RegDef_t* gives direct field access to CR1, CR2, OAR1, OAR2, DR, SR1, SR2, CCR, TRISE, and FLTR.

Figure 4. Class Diagram, I2C_RegDef_t struct, the in-memory layout of a single I2C peripheral. The order and width of the fields exactly match RM0383 §18.6.

The driver also defines a configuration struct, i2c_config_t, that the caller fills in before initialization. It groups the three settings i2c_init needs into one object, which keeps the function signature short and makes the call site self-documenting.

Figure 5. Class Diagram, i2c_config_t configuration struct, passed to i2c_init.

Finally, the driver defines the i2c_status_t enumeration, which acts as the return type of every public function. It distinguishes success (I2C_OK) from the specific failure modes the bus can produce: an invalid argument, a busy bus, a device NACK, a timeout, and a bus error.

3.2. Module Behavior (Sequence Diagrams, State Machines, etc.)

This subsection describes the runtime behavior of the driver. Because I2C is a sequenced protocol, every transfer function is effectively a small state machine that walks through bus conditions in a fixed order, waiting on a status flag at each step.

The initialization path (the equivalent of the template's Driver Init activity) is straight-line: i2c_init enables the peripheral clock, performs a software reset to clear any stuck state, programs CR2 with the APB1 frequency, computes and writes CCR and TRISE for the requested speed, and finally sets PE to bring the peripheral online. There are no branches; either the arguments are valid and the peripheral comes up, or the function rejects a NULL or zero argument up front.

A register read (the equivalent of the template's Driver Action with Conditional and Loop) is the most involved path. i2c_readRegDevice generates a START, sends the device address with the write bit, sends the target register, generates a repeated START, re-sends the address with the read bit, and then loops reading bytes from DR. At each step it waits on the relevant flag (SB, then ADDR, then RXNE) inside a bounded loop, and it checks for the AF (NACK) and BERR (bus error) flags so it can abort cleanly with the matching status code. The final byte is NACKed deliberately, which is how an I2C master tells the slave to stop sending, followed by a STOP condition.

A write (i2c_writeRegDevice) follows the same shape but simpler: START, address with write bit, register, then a loop pushing bytes into DR while waiting on TXE and BTF, and finally a STOP. The raw variants (i2c_writeDevice, i2c_readDevice) are the same minus the register-address step, for devices that do not present an internal register map.

3.3. I2C Driver

The driver is structured around a validate-then-sequence pattern. Every public function first checks its arguments (NULL instance or buffer, zero length) and returns I2C_ERR_INVALID_ARG before touching the bus. It then checks that the bus is not already busy (SR2.BUSY) and returns I2C_ERR_BUS_BUSY if it is, which prevents one transfer from stomping on another. Only after those checks does it begin generating bus conditions.

Two implementation choices are worth highlighting because they have non-obvious consequences for users of the driver:

- Bounded polling on every flag wait. Each wait for a status flag (SB, ADDR, TXE, RXNE, BTF) is wrapped in a countdown initialized to I2C_TIMEOUT_MAX. On a healthy bus the flag asserts within a handful of cycles, but if a device is missing or a wire is loose, an unbounded loop would freeze the whole program. The countdown turns that silent hang into an explicit I2C_ERR_TIMEOUT return. This is the same defensive pattern used in the UART driver from the previous lab.

- Distinct error codes for distinct failures. The driver separates a NACK (I2C_ERR_NACK, the device answered and refused) from a timeout (I2C_ERR_TIMEOUT, the device never answered) from a bus error (I2C_ERR_BUS_ERROR, a misplaced START or STOP). During bring-up these three failures have completely different causes, so collapsing them into one code would have made debugging much harder. Keeping them separate is what let us quickly tell the difference between a wrong address and a disconnected sensor.

One subtlety inherited from the hardware is the ADDR-clear sequence. On the STM32 I2C peripheral, the ADDR flag is not cleared by writing to it; it is cleared by reading SR1 followed by SR2. The driver performs that read pair at the right moment in every transfer, because skipping it leaves the peripheral stalled waiting for software to acknowledge the address phase. This is one of the details that the reference manual specifies precisely and that cannot be guessed from the protocol alone.

4. Application Programming Interfaces

4.1. Data Types and Macros

The driver defines the following public types and macros in i2c_driver.h.

Structures

I2C_RegDef_t: Memory-mapped register layout for one I2C peripheral. Casting a base address to (I2C_RegDef_t *) gives field access to all registers. Layout matches RM0383 §18.6.

| Member Name | Type | Notes/Definition |
| --- | --- | --- |
| CR1 | uint32_t | 0x00 - Control register 1 (PE, START, STOP, ACK, SWRST). |
| CR2 | uint32_t | 0x04 - Control register 2 (peripheral clock frequency in MHz). |
| OAR1 | uint32_t | 0x08 - Own address register 1 (used in slave mode). |
| OAR2 | uint32_t | 0x0C - Own address register 2 (dual addressing). |
| DR | uint32_t | 0x10 - Data register (write to transmit, read to receive). |
| SR1 | uint32_t | 0x14 - Status register 1 (event and error flags). |
| SR2 | uint32_t | 0x18 - Status register 2 (BUSY, MSL, TRA flags). |
| CCR | uint32_t | 0x1C - Clock control register (SCL timing). |
| TRISE | uint32_t | 0x20 - Maximum rise-time register. |
| FLTR | uint32_t | 0x24 - Digital noise filter register. |

i2c_config_t: Run-time configuration passed to i2c_init.

| Member Name | Type | Notes/Definition |
| --- | --- | --- |
| instance | I2C_RegDef_t* | Peripheral pointer: I2C1, I2C2, or I2C3. |
| clk_speed | uint32_t | Bus speed in Hz: I2C_SPEED_SM_HZ (100k) or I2C_SPEED_FM_HZ (400k). |
| addr_mode | uint8_t | I2C_ADDR_7BIT or I2C_ADDR_10BIT. |

Macros

| Member Name | Type | Notes/Definition |
| --- | --- | --- |
| I2C1 / I2C2 / I2C3 | I2C_PERIPH(base) | Typed pointers to the three I2C peripheral register blocks. |
| I2C_SPEED_SM_HZ | 100000U | Standard-mode bus speed, 100 kHz. |
| I2C_SPEED_FM_HZ | 400000U | Fast-mode bus speed, 400 kHz. |
| I2C_ADDR_7BIT | 0U | Selects 7-bit device addressing. |
| I2C_ADDR_10BIT | 1U | Selects 10-bit device addressing. |
| I2C_TIMEOUT_MAX | 1000000U | Polling iteration limit before a transfer is declared timed-out (NFR-3). |

4.2. Error Codes

The I2C APIs have a return type of i2c_status_t. A return value of I2C_OK indicates success; any non-zero value indicates a specific class of failure.

| Code | Notes/Definition |
| --- | --- |
| I2C_OK | 0 - Operation completed successfully. |
| I2C_ERR_INVALID_ARG | -1 - A NULL pointer or illegal parameter was supplied (NULL instance/buffer, zero length, or zero clock speed). |
| I2C_ERR_BUS_BUSY | -2 - The bus was not idle (SR2.BUSY set) when a START was requested. |
| I2C_ERR_NACK | -3 - The addressed device did not acknowledge (SR1.AF set). Usually a wrong address or an absent device. |
| I2C_ERR_TIMEOUT | -4 - A flag wait exceeded I2C_TIMEOUT_MAX. The device never responded. |
| I2C_ERR_BUS_ERROR | -5 - A misplaced START or STOP was detected on the bus (SR1.BERR set). |

4.3. Function Definitions

This section documents the five public functions exposed by the I2C driver. Each table follows the standard SDD format.

i2c_init

| Function Name | i2c_init |  |
| --- | --- | --- |
| Syntax | i2c_status_t i2c_init(const i2c_config_t *config); |  |
| Parameters (in) | config | Pointer to a populated i2c_config_t. Must not be NULL; instance must be I2C1/2/3; clk_speed must be non-zero. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | i2c_status_t | I2C_OK on success; I2C_ERR_INVALID_ARG if config or instance is NULL, or clk_speed is zero. |
| Description | Initializes the I2C peripheral (FR-1, FR-2). Enables the APB1 clock for the selected instance, performs a software reset (SWRST) to clear any previous fault state, programs CR2 with the APB1 frequency in MHz, computes and writes CCR for the requested bus speed, computes and writes TRISE, and finally sets PE to leave the peripheral ready. |  |
| Constraints | The SCL and SDA GPIO pins must already be configured as open-drain alternate-function outputs with pull-ups enabled, by the caller via the GPIO driver, before this function has any visible effect on the bus. |  |
| Available via | i2c_driver.h |  |

i2c_writeRegDevice

| Function Name | i2c_writeRegDevice |  |
| --- | --- | --- |
| Syntax | i2c_status_t i2c_writeRegDevice(I2C_RegDef_t *instance, uint16_t dev_addr, uint8_t reg_addr, const uint8_t *data, size_t len); |  |
| Parameters (in) | instance | Peripheral pointer initialized by i2c_init. |
|  | dev_addr | Device address, unshifted (7-bit: 0x00-0x7F). |
|  | reg_addr | Target register inside the device. |
|  | data | Bytes to transmit. Must not be NULL. |
|  | len | Number of bytes (>= 1). |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | i2c_status_t | I2C_OK on success; I2C_ERR_INVALID_ARG, I2C_ERR_BUS_BUSY, I2C_ERR_NACK, I2C_ERR_TIMEOUT, or I2C_ERR_BUS_ERROR on the corresponding conditions. |
| Description | Writes one or more bytes to a register of an I2C device (FR-3, FR-4). Bus sequence: START, address with write bit, register address, data bytes, STOP. |  |
| Constraints | instance must have been initialized by i2c_init. The bus must be idle at START. The function blocks until completion or timeout. |  |
| Available via | i2c_driver.h |  |

i2c_writeDevice

| Function Name | i2c_writeDevice |  |
| --- | --- | --- |
| Syntax | i2c_status_t i2c_writeDevice(I2C_RegDef_t *instance, uint16_t dev_addr, const uint8_t *data, size_t len); |  |
| Parameters (in) | instance | Peripheral pointer initialized by i2c_init. |
|  | dev_addr | Device address, unshifted. |
|  | data | Bytes to transmit. Must not be NULL. |
|  | len | Number of bytes (>= 1). |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | i2c_status_t | Same set of codes as i2c_writeRegDevice. |
| Description | Writes bytes directly to an I2C device without a leading register address (FR-5, FR-6). Bus sequence: START, address with write bit, data bytes, STOP. Used for devices that do not present an internal register map. |  |
| Constraints | instance must have been initialized by i2c_init. The bus must be idle at START. |  |
| Available via | i2c_driver.h |  |

i2c_readRegDevice

| Function Name | i2c_readRegDevice |  |
| --- | --- | --- |
| Syntax | i2c_status_t i2c_readRegDevice(I2C_RegDef_t *instance, uint16_t dev_addr, uint8_t reg_addr, uint8_t *buf, size_t len); |  |
| Parameters (in) | instance | Peripheral pointer initialized by i2c_init. |
|  | dev_addr | Device address, unshifted. |
|  | reg_addr | Source register inside the device. |
|  | len | Number of bytes to read (>= 1). |
| Parameters (inout) | None |  |
| Parameters (out) | buf | Receive buffer. Must not be NULL and must hold at least len bytes. |
| Return value | i2c_status_t | I2C_OK on success; the same error codes as the write functions on the corresponding conditions. |
| Description | Reads one or more bytes from a register of an I2C device (FR-7, FR-8). Bus sequence: START, address with write bit, register address, repeated START, address with read bit, data bytes, STOP. The last byte read is NACKed to signal the slave to stop. |  |
| Constraints | instance must have been initialized by i2c_init. The ADDR flag is cleared by the mandated SR1-then-SR2 read sequence inside the function. |  |
| Available via | i2c_driver.h |  |

i2c_readDevice

| Function Name | i2c_readDevice |  |
| --- | --- | --- |
| Syntax | i2c_status_t i2c_readDevice(I2C_RegDef_t *instance, uint16_t dev_addr, uint8_t *buf, size_t len); |  |
| Parameters (in) | instance | Peripheral pointer initialized by i2c_init. |
|  | dev_addr | Device address, unshifted. |
|  | len | Number of bytes to read (>= 1). |
| Parameters (inout) | None |  |
| Parameters (out) | buf | Receive buffer. Must not be NULL. |
| Return value | i2c_status_t | Same set of codes as i2c_readRegDevice. |
| Description | Reads bytes directly from an I2C device without a leading register address (FR-9, FR-10). Bus sequence: START, address with read bit, data bytes, STOP. |  |
| Constraints | instance must have been initialized by i2c_init. The bus must be idle at START. |  |
| Available via | i2c_driver.h |  |

4.4. Internal Function Definitions

The I2C driver keeps its flag-wait and bus-condition logic inline inside the five public functions rather than exposing separate static helpers, so there are no internal functions with their own linkage. Two patterns recur and behave conceptually as internal subroutines, and are documented here as candidates for future refactoring.

wait_flag (inline pattern)

| Function Name | wait_flag (currently inlined) |  |
| --- | --- | --- |
| Syntax | /* Inlined bounded poll on an SR1/SR2 flag */ |  |
| Parameters (in) | flag | The status-register bit to wait for (SB, ADDR, TXE, RXNE, BTF). |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | i2c_status_t | I2C_OK if the flag asserts within I2C_TIMEOUT_MAX; I2C_ERR_TIMEOUT otherwise. |
| Description | Implements the bounded wait applied before every bus step. Also checks the AF and BERR error flags so a NACK or bus error aborts the transfer immediately with the correct code. |  |
| Constraints | Duplicated inline across all four transfer functions. Extracting it into a static helper would reduce code size and centralize the timeout policy. |  |
| Available via | i2c_driver.c (not declared in any header, no external linkage) |  |

clear_addr (inline pattern)

| Function Name | clear_addr (currently inlined) |  |
| --- | --- | --- |
| Syntax | /* Inlined SR1-then-SR2 read to clear ADDR */ |  |
| Parameters (in) | instance | The I2C peripheral whose ADDR flag must be cleared. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | void | No return; performs the mandated register read pair. |
| Description | Clears the ADDR flag by reading SR1 followed by SR2, exactly as RM0383 requires. Skipping this leaves the peripheral stalled in the address phase. |  |
| Constraints | Must be performed at the correct point in every transfer. The read result is intentionally discarded. |  |
| Available via | i2c_driver.c (not declared in any header, no external linkage) |  |

5. Future Enhancements

The current driver covers the requirements of this lab, but several extensions are natural next steps. The list below is ordered roughly by priority and effort.

- Interrupt-driven and DMA transfers. The driver is purely polled, so the CPU busy-waits on every flag. Enabling the event and error interrupts (ITEVTEN, ITERREN in CR2), or wiring a DMA stream, would let the application keep working while a transfer runs in the background.

- 10-bit addressing support. The addr_mode field already accepts I2C_ADDR_10BIT, but the transfer functions currently implement only the 7-bit address phase. Completing the 10-bit path would let the driver talk to the small number of devices that use extended addressing.

- Internal helper extraction. As documented in Section 4.4, the flag-wait and ADDR-clear patterns are duplicated across the transfer functions. Factoring them into static helpers would reduce code size and centralize future changes to the timeout and error policy.

- Bus-recovery routine. If a slave hangs while holding SDA low, the bus can lock up. A recovery routine that manually toggles SCL to free a stuck slave, then re-initializes the peripheral, would make the driver more robust in the field.

- Repeated-start chaining for combined transactions. Exposing a lower-level API that lets a caller chain several reads and writes under a single START/STOP would let device drivers implement multi-step protocols (such as some EEPROM page operations) more efficiently.

- Configurable digital noise filter. The FLTR register is left at its reset value. Exposing it through i2c_config_t would let noisy or long bus installations tune out spurious spikes.