1. Introduction

1.1. Purpose

This document describes the design and architecture of the SPI driver module, based on the requirements defined in the SRS-SPI_Driver. It provides details on data structures, register-level operations, control flow, and the public API used by higher-level modules. The intent is to give any developer reading this document enough context to understand, maintain, extend, or replicate the driver without having to reverse-engineer the source code.

The driver targets the STM32F411RE microcontroller mounted on the NUCLEO-64 development board, and exposes a hardware-abstracted API that hides the manipulation of the SPI peripheral registers (CR1, CR2, SR, DR) and the relevant clock-enable bits of RCC_APB1ENR / RCC_APB2ENR from the rest of the application. Chip-select handling is delegated entirely to the existing GPIO driver, so the SPI driver itself never owns a GPIO register.

1.2. Scope

The SPI driver is a low-level software component for embedded systems running on the STM32F411RE. It abstracts the SPI hardware registers documented in RM0383 §20 and provides a clean, uniform API for higher-level modules to initialize the peripheral in master mode, transmit and receive byte buffers, and drive a chip-select line, without ever touching a SPI register directly.

The driver supports the SPI peripherals present on the F411 (SPI1 through SPI5) through an instance pointer, configurable clock prescaler, all four SPI modes (CPOL/CPHA combinations), 8- or 16-bit data frames, and MSB- or LSB-first bit order. In this lab the driver was implemented and tested as a standalone deliverable; the integrated demo used the I2C sensor instead, since the TCS3472 is an I2C part. The only header dependencies outside of standard C are the device-specific register layout in spi_driver.h and the GPIO driver header used for chip-select control.

1.3. Definitions, Acronyms, and Abbreviations

| Term | Definition |
| --- | --- |
| SPI | Serial Peripheral Interface. A four-wire, full-duplex synchronous bus (SCK, MOSI, MISO, plus a per-device chip-select). |
| MCU | Microcontroller Unit. The STM32F411RE in this project. |
| RCC | Reset and Clock Control. Gates the clock to every peripheral on the chip. |
| APB1 / APB2 | Advanced Peripheral Buses. SPI2/SPI3 live on APB1; SPI1/SPI4/SPI5 live on APB2. |
| SCK | Serial Clock. Driven by the master on every transfer. |
| MOSI / MISO | Master Out Slave In / Master In Slave Out. The two unidirectional data lines. |
| CS / NSS | Chip Select (also Slave Select). An active-low line that selects which device the master is talking to. |
| CPOL / CPHA | Clock Polarity / Clock Phase. The two settings that define the four SPI modes. |
| CR1 / CR2 | Control Registers. CR1 holds MSTR, SPE, BR, CPOL, CPHA, DFF, LSBFIRST, SSM, SSI; CR2 holds buffer and interrupt enables. |
| SR | Status Register. Holds the TXE, RXNE, BSY, and OVR flags. |
| DR | Data Register. Writing queues a byte to transmit; reading returns a received byte. |
| TXE | Transmit buffer Empty flag, set when DR can accept a new byte. |
| RXNE | Receive buffer Not Empty flag, set when DR holds a received byte. |
| BSY | Busy flag, set while a transfer is in progress. |
| OVR | Overrun flag, set when a byte arrives before the previous one was read. |
| DFF | Data Frame Format, selects an 8-bit or 16-bit frame. |
| SRS | Software Requirements Specification. |
| SDD | Software Design Document, this document. |
| FR / NFR | Functional / Non-Functional Requirement, as listed in the SRS. |
| API | Application Programming Interface. The set of public functions exposed by the driver. |

1.4. Requirements

All requirements specific to this module are allocated in SRS-SPI_Driver.md, available in the lab repository at https://github.com/MasterCarlosV/MSE_LabCourse/blob/main/SRS/SPI/SRS-SPI_Driver.md. The driver implements functional requirements FR-1 through FR-8 and complies with the non-functional requirement on error handling, with every public function returning a status code of type SPI_Status_t.

2. System Architecture

2.1. High-Level Design

The system follows a classic layered architecture, where each layer talks only to the one immediately below it. The SPI driver is unusual among the drivers in this project in that it depends on another driver, the GPIO driver, for its chip-select handling, rather than owning that hardware itself.

- Application / Device Layer. A device driver (for a SPI display, ADC, or memory chip) would call spi_transmit, spi_receive, and the chip-select functions to talk to its part. In this lab no SPI device was integrated, so this layer is represented by the standalone tests used to validate the driver.

- SPI Driver Layer, spi_driver.c/.h. Implements the public API and is the only layer allowed to touch the SPI peripheral registers and the relevant bits of RCC_APBxENR.

- GPIO Driver Layer, GPIO_stm32.c/.h. Reused from the earlier labs. The SPI driver calls gpio_setPin and gpio_clearPin through it to drive the chip-select line, instead of duplicating pin-handling code.

- Hardware Layer, STM32F411RE SPI peripheral registers, mapped at fixed addresses (SPI1 base 0x40013000 on APB2, SPI2/SPI3 on APB1).

Figure 1 shows the headers that spi_driver.h includes. Unlike the other drivers, it pulls in GPIO_stm32.h, which is what lets the chip-select functions reuse the GPIO driver's pin types and operations rather than redefining them.

Figure 1. Headers included by spi_driver.h. The dependency on GPIO_stm32.h is what allows chip-select reuse.

2.2. Module Interfaces

The public API is defined in spi_driver.h. It exposes five functions: one for initialization, one each for transmit and receive, and two for chip-select control. Module-level dependencies are the C standard headers (stdint.h, stddef.h) plus the GPIO driver header. There is no dependency on STM32 HAL, CMSIS device headers, or any third-party library.

Figure 2 shows the inclusion graph of main.c for context, with the modules used in the integrated demo. The SPI driver is not part of that graph because the demo ran over I2C; the SPI driver was exercised through its own standalone tests instead.

Figure 2. Inclusion graph for main.c, showing the modules used in the integrated demo. The SPI driver was validated separately.

3. Detailed Design

3.1. Class Diagram

Although C is not an object-oriented language, the SPI_RegDef_t struct plays the role of a class in this design: it groups together all the registers that belong to a single SPI peripheral and lets us treat the whole peripheral as a single addressable object. Casting any SPI base address (0x40013000 for SPI1, etc.) to a SPI_RegDef_t* gives direct field access to CR1, CR2, SR, DR, CRCPR, RXCRCR, TXCRCR, I2SCFGR, and I2SPR.

Figure 3. Class Diagram, SPI_RegDef_t struct, the in-memory layout of a single SPI peripheral. The order and width of the fields exactly match RM0383 §20.5.

The driver defines a configuration struct, SPI_Config_t, holding the four settings spi_init needs: the baud-rate prescaler, the SPI mode (CPOL/CPHA), the data frame format, and the bit order. Grouping them keeps the init signature short and the call site readable.

Figure 4. Class Diagram, SPI_Config_t configuration struct, passed to spi_init.

Finally, the SPI_CS_t descriptor pairs a GPIO port with a pin number, using the GPIO driver's public types. This is what lets spi_csEnable and spi_csDisable operate any chip-select pin on any port without hard-coding an address.

Figure 5. Class Diagram, SPI_CS_t chip-select descriptor, built from the GPIO driver's port and pin types.

3.2. Module Behavior (Sequence Diagrams, State Machines, etc.)

This subsection describes the runtime behavior of the driver. SPI is simpler than I2C because there are no addresses, acknowledge bits, or start/stop conditions; a transfer is just shifting bytes while a flag tells you when the buffer is ready.

The initialization path (the equivalent of the template's Driver Init activity) is straight-line: spi_init validates its arguments, enables the peripheral clock on the correct bus (APB1 or APB2 depending on the instance), programs CR1 with the prescaler, mode, frame format, and bit order, configures software slave management, and sets SPE to enable the peripheral. Either the arguments are valid and the peripheral comes up, or the function rejects a NULL pointer or invalid parameter up front.

A transmit (the equivalent of the template's Driver Action with Loop) writes each byte into DR after waiting for the TXE flag, then waits for the BSY flag to clear at the end so the caller knows the last byte has fully left the shift register. A receive is the mirror image: it writes a dummy byte to generate the clock (SPI has no separate read strobe), waits for RXNE, and reads the incoming byte from DR, repeating for each byte requested. Both loops check their flags inside a bounded countdown so a stuck bus returns SPI_ERR_TIMEOUT instead of hanging.

The chip-select functions are the equivalent of the template's Driver Action with Conditional: each validates its descriptor pointer, then calls down into the GPIO driver (gpio_clearPin to assert CS low, gpio_setPin to release it high) and translates the GPIO driver's return code into a SPI status code.

3.3. SPI Driver

The driver is structured around a validate-then-transfer pattern. Every public function first checks its arguments (NULL SPI pointer, NULL buffer, zero length) and returns the matching error before touching the bus. Transfers then poll the relevant flag at each step inside a bounded loop.

Two implementation choices are worth highlighting because they have non-obvious consequences for users of the driver:

- Chip-select reuse through the GPIO driver. Rather than implementing its own pin control, the SPI driver carries a small SPI_CS_t descriptor and calls gpio_setPin / gpio_clearPin. This keeps the SPI driver focused purely on the bus and means any pin on any port can serve as a chip-select with no changes to the SPI code. It also means the chip-select functions can return SPI_ERR_GPIO when the underlying GPIO call fails, surfacing a misconfigured pin clearly.

- Bounded polling with overrun detection. Every flag wait (TXE, RXNE, BSY) is wrapped in a countdown limited by SPI_TIMEOUT, the same defensive pattern used in the UART and I2C drivers. The receive path additionally checks the OVR flag, returning SPI_ERR_OVERRUN if a byte arrived before the previous one was read, which is the classic failure mode of a polled SPI receiver that falls behind.

One design point worth noting is that chip-select is deliberately not automatic. spi_transmit and spi_receive move bytes but do not touch the CS line, leaving the caller to bracket a transaction with spi_csEnable and spi_csDisable. This is intentional: many SPI devices require several transfers (a command followed by data) under a single continuous CS assertion, and an automatic per-call CS toggle would make those multi-step transactions impossible. Giving the caller explicit control is the more flexible design.

4. Application Programming Interfaces

4.1. Data Types and Macros

The driver defines the following public types and macros in spi_driver.h.

Structures

SPI_RegDef_t: Memory-mapped register layout for one SPI peripheral. Layout matches RM0383 §20.5.

| Member Name | Type | Notes/Definition |
| --- | --- | --- |
| CR1 | uint32_t | 0x00 - Control register 1 (MSTR, SPE, BR, CPOL, CPHA, DFF, LSBFIRST, SSM, SSI). |
| CR2 | uint32_t | 0x04 - Control register 2 (buffer and interrupt enables). |
| SR | uint32_t | 0x08 - Status register (TXE, RXNE, BSY, OVR flags). |
| DR | uint32_t | 0x0C - Data register (write to transmit, read to receive). |
| CRCPR | uint32_t | 0x10 - CRC polynomial register. |
| RXCRCR | uint32_t | 0x14 - RX CRC register (read-only). |
| TXCRCR | uint32_t | 0x18 - TX CRC register (read-only). |
| I2SCFGR | uint32_t | 0x1C - I2S configuration register (unused in SPI mode). |
| I2SPR | uint32_t | 0x20 - I2S prescaler register (unused in SPI mode). |

SPI_Config_t: Run-time configuration passed to spi_init.

| Member Name | Type | Notes/Definition |
| --- | --- | --- |
| baudRate | uint32_t | Clock prescaler, one of SPI_BAUD_DIV2 through SPI_BAUD_DIV256. |
| mode | uint32_t | SPI mode, one of SPI_MODE_0 through SPI_MODE_3 (CPOL/CPHA). |
| dataFrameFormat | uint32_t | Frame size, SPI_DFF_8BIT or SPI_DFF_16BIT. |
| firstBit | uint32_t | Bit order, SPI_FIRSTBIT_MSB or SPI_FIRSTBIT_LSB. |

SPI_CS_t: Chip-select descriptor, pairing a GPIO port and pin using the GPIO driver's public types.

| Member Name | Type | Notes/Definition |
| --- | --- | --- |
| port | gpio_port_t | GPIO port of the chip-select pin. |
| pin | gpio_pin_t | Pin number of the chip-select line. |

Macros

| Member Name | Type | Notes/Definition |
| --- | --- | --- |
| SPI1..SPI5_BASE_ADDR | address | Base addresses of the five SPI peripherals (SPI1/4/5 on APB2, SPI2/3 on APB1). |
| SPI_BAUD_DIV2..DIV256 | CR1[5:3] | Baud-rate prescaler selectors, fPCLK divided by 2 up to 256. |
| SPI_MODE_0..3 | CPOL/CPHA | The four SPI modes formed by combining clock polarity and phase. |
| SPI_DFF_8BIT / 16BIT | CR1.DFF | Selects an 8-bit or 16-bit data frame. |
| SPI_FIRSTBIT_MSB / LSB | CR1.LSBFIRST | Selects MSB-first or LSB-first bit order. |
| SPI_TIMEOUT | 100000U | Polling iteration limit before a flag wait returns SPI_ERR_TIMEOUT. |

4.2. Error Codes

The SPI APIs have a return type of SPI_Status_t. A return value of SPI_OK indicates success; any non-zero value indicates a specific class of failure.

| Code | Notes/Definition |
| --- | --- |
| SPI_OK | 0x00 - Operation completed successfully. |
| SPI_ERR_NULL_PTR | 0x01 - A NULL pointer was passed as a buffer or descriptor argument. |
| SPI_ERR_BUSY | 0x02 - The SPI bus was busy when an operation was requested. |
| SPI_ERR_OVERRUN | 0x03 - The OVR flag was set during reception; a byte was lost. |
| SPI_ERR_TIMEOUT | 0x04 - A flag wait (TXE/RXNE/BSY) exceeded SPI_TIMEOUT. |
| SPI_ERR_PARAM | 0x05 - An invalid parameter was supplied (for example, length equal to zero). |
| SPI_ERR_GPIO | 0x06 - The GPIO driver returned an error during a chip-select operation. |

4.3. Function Definitions

This section documents the five public functions exposed by the SPI driver. Each table follows the standard SDD format.

spi_init

| Function Name | spi_init |  |
| --- | --- | --- |
| Syntax | SPI_Status_t spi_init(SPI_RegDef_t *spi, const SPI_Config_t *config); |  |
| Parameters (in) | spi | Pointer to the target SPI register block (SPI_PERIPH(SPI1_BASE_ADDR), etc.). |
|  | config | Pointer to a fully initialized SPI_Config_t. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | SPI_Status_t | SPI_OK on success; SPI_ERR_NULL_PTR if spi or config is NULL; SPI_ERR_PARAM if a config field is invalid. |
| Description | Initializes the SPI peripheral in master mode (FR-1, FR-2). Enables the peripheral clock in the correct RCC_APBxENR register, programs CR1 with the prescaler, mode, frame format, and bit order, configures software slave management, and sets SPE to enable the peripheral. |  |
| Constraints | The SPI SCK/MOSI/MISO pins must be configured as alternate-function by the caller via the GPIO driver before this call. The chip-select pin must be configured as a GPIO output separately. |  |
| Available via | spi_driver.h |  |

spi_transmit

| Function Name | spi_transmit |  |
| --- | --- | --- |
| Syntax | SPI_Status_t spi_transmit(SPI_RegDef_t *spi, const uint8_t *buffer, size_t length); |  |
| Parameters (in) | spi | Pointer to the SPI register block. |
|  | buffer | Source buffer. Must not be NULL and must hold at least length bytes. |
|  | length | Number of bytes to transmit. Must be > 0. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | SPI_Status_t | SPI_OK on success; SPI_ERR_NULL_PTR if spi or buffer is NULL; SPI_ERR_PARAM if length is zero; SPI_ERR_TIMEOUT if TXE or BSY did not clear in time. |
| Description | Transmits a buffer of bytes over the SPI bus (FR-3, FR-4). Writes each byte into DR after polling TXE, then waits for BSY to clear at the end so the caller knows the final byte has fully shifted out. |  |
| Constraints | spi must have been initialized by spi_init. The function does not assert or release the chip-select line; the caller brackets the transaction with spi_csEnable / spi_csDisable. |  |
| Available via | spi_driver.h |  |

spi_receive

| Function Name | spi_receive |  |
| --- | --- | --- |
| Syntax | SPI_Status_t spi_receive(SPI_RegDef_t *spi, uint8_t *buffer, size_t length); |  |
| Parameters (in) | spi | Pointer to the SPI register block. |
|  | length | Number of bytes to receive. Must be > 0. |
| Parameters (inout) | None |  |
| Parameters (out) | buffer | Destination buffer. Must not be NULL and must hold at least length bytes. |
| Return value | SPI_Status_t | SPI_OK on success; SPI_ERR_NULL_PTR if spi or buffer is NULL; SPI_ERR_PARAM if length is zero; SPI_ERR_OVERRUN on an OVR event; SPI_ERR_TIMEOUT if RXNE did not assert in time. |
| Description | Receives a buffer of bytes over the SPI bus (FR-5, FR-6). For each byte it writes a dummy value to generate the SPI clock, polls RXNE, and reads the incoming byte from DR. |  |
| Constraints | spi must have been initialized by spi_init. As with transmit, the caller manages the chip-select line. The OVR flag is checked to detect a receiver that fell behind. |  |
| Available via | spi_driver.h |  |

spi_csEnable

| Function Name | spi_csEnable |  |
| --- | --- | --- |
| Syntax | SPI_Status_t spi_csEnable(const SPI_CS_t *cs); |  |
| Parameters (in) | cs | Pointer to a fully initialized SPI_CS_t descriptor. The pin must already be configured as a GPIO output. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | SPI_Status_t | SPI_OK if the CS pin was driven low; SPI_ERR_NULL_PTR if cs is NULL; SPI_ERR_GPIO if gpio_clearPin returned an error. |
| Description | Asserts (drives low) the chip-select line to select the target SPI device (FR-7). Internally calls gpio_clearPin(cs->port, cs->pin) on the GPIO driver, since CS is active-low. |  |
| Constraints | The pin must have been configured as GPIO_MODE_OUTPUT via gpio_setPinMode before any call. |  |
| Available via | spi_driver.h |  |

spi_csDisable

| Function Name | spi_csDisable |  |
| --- | --- | --- |
| Syntax | SPI_Status_t spi_csDisable(const SPI_CS_t *cs); |  |
| Parameters (in) | cs | Pointer to a fully initialized SPI_CS_t descriptor. |
| Parameters (inout) | None |  |
| Parameters (out) | None |  |
| Return value | SPI_Status_t | SPI_OK if the CS pin was driven high; SPI_ERR_NULL_PTR if cs is NULL; SPI_ERR_GPIO if gpio_setPin returned an error. |
| Description | Deasserts (drives high) the chip-select line to release the SPI device (FR-8). Internally calls gpio_setPin(cs->port, cs->pin) on the GPIO driver. |  |
| Constraints | The pin must have been configured as GPIO_MODE_OUTPUT before any call. |  |
| Available via | spi_driver.h |  |

4.4. Internal Function Definitions

The SPI driver does not define any internal (static) helper functions. The transmit and receive functions are short bounded-polling loops, and the chip-select functions are thin wrappers over single GPIO calls, so no shared logic emerged that would justify a separate helper.

The one pattern that recurs is the bounded flag wait (a countdown on TXE, RXNE, or BSY), which appears inline in spi_transmit and spi_receive. If the driver grew to support interrupt-driven or DMA transfers, factoring that wait into a static helper would be the natural first refactoring.

5. Future Enhancements

The current driver covers the requirements of this lab, but several extensions are natural next steps. The list below is ordered roughly by priority and effort.

- Full-duplex transfer. spi_transmit and spi_receive are separate half-duplex calls. A spi_transferDuplex(tx, rx, len) that shifts a byte out and reads a byte in on the same clock would use the bus more efficiently and match how many SPI devices (such as ADCs) actually work.

- Interrupt-driven and DMA transfers. The driver is purely polled. Enabling the TXE and RXNE interrupts in CR2, or wiring the SPI to a DMA stream, would let the CPU do other work during long transfers, which matters for displays and memory chips that move large blocks.

- 16-bit frame transfers. The DFF setting is configurable in SPI_Config_t, but the transmit and receive loops currently move one byte at a time. Completing the 16-bit path would let the driver talk to devices that use 16-bit words natively without the caller splitting them by hand.

- Optional automatic chip-select. While explicit CS control is the more flexible default, an optional convenience wrapper that asserts CS, runs a transfer, and releases CS in one call would simplify the common single-transaction case for device drivers that do not need continuous assertion.

- Hardware NSS and multi-master support. The driver uses software slave management (SSM). Adding hardware NSS output mode and the groundwork for multi-master arbitration would broaden the range of bus topologies the driver can handle.

- CRC support. The peripheral has built-in CRC hardware (CRCPR, RXCRCR, TXCRCR) that the driver leaves unused. Exposing it would let the driver verify data integrity on noisy links, which is useful for memory and safety-critical devices.